1. In LabVIEW, deactivate controllers CV101, CV102 & CV103. Make sure the valves remain opened using the control override

EROSION SHUTDOWN PROCEDURE

2. Open recycle valve V107

3. Using the control override, close CV101, CV102 & CV103

4. Close air valves FIC104, FIC105 & FIC106

5. Manually close V102, V103 & V104

6. Remove the probe + support from the erosion boxes as follows:

a) Remove the black cover and open the box using the wingnuts

b) Use an L-shaped tool to remove the probe + support (you can find it inside the toolbox)

c) Close the erosion box using the wingnuts

7. Follow the shutdown procedure

8. Un-plug the light source

Page 1/1									END