1. Follow the start up procedure all the way to the end

EROSION STARTUP PROCEDURE

2. Prepare the probe as follows:

- Get the probe support from the cabinet

- Place the sample in the support

- Fixate the probe using a screw (M4-30mm) and a nut

3. Plug in the light source (on the wall to the left)

4. Test the cameras (see Camera Setting Procedure)

5. In LabVIEW, deactivate controllers CV101, CV102 & CV103.

Make sure the valves remain opened using the control override

6. Open recycle valve V107

7. Using the control override, close CV101, CV102 & CV103

8. Close air valves FIC104, FIC105 & FIC106

9. Manually close V102, V103 & V104

8.

7.

6.

Page 1/2

10. Insert the probe in the erosion boxes as follows:

a) Open the box using the wingnuts

b) Place the probe + support with the big hole facing the inlet

c) Close the box using the wingnuts and put the black cover on it

11. In LabVIEW, open CV101, CV102 & CV103

12. Open V102, V103 & V104

(any order)

13. If the flow is normal, close the recycle valve V107

V102, V103, V104

V107

page 2/2					END